DVRAS – Digital VLF Recording and Analysis System

The analysis of VLF wave data recorded at high latitudes is a powerful tool for remote sensing of processes in the magnetosphere, providing data complementary to other techniques which, for example, monitor particle precipitation and magnetic field variations. This manual describes the Digitised VLF Recording and Analysis System (DVRAS) developed at the Space Physics Research Institute (SPRI).

The DVRAS system is comprised of the following components: antenna, pre-amplifier, goniometer, programmable power supply, signal attenuator, GPS, and a workstation, arranged as indicated in Figure 2.1. Broadband VLF signals from the pair of orthogonal vertical loop antennae pass through the preamplifier (located at the antenna site). The Digital Very Low Frequency (VLF) Recording and Analysis System (DVRAS) is used to collect this VLF wave data.

Figure 1. Diagram of the connections to the DVRAS system.

The signal acquired at the antenna is boosted by the pre-amplifier before passing via a pair of lengthy coaxial cables to the control unit where it is anti-alias filtered before being digitised through the workstation’s sound card. The signal conditioner (VLF Receiver) simply scales the input signal to a range suitable for input to the sound card. The time source (GPS or radio clock) is used to accurately timestamp the digitised data. The workstation’s parallel port interface is used to periodically activate a module in the control unit which generates a set of calibration tones which are fed back into the system through a small coil mounted at the centre of the antenna.

dvrecord

The capture of digitised data is conducted by the dvrec program. The execution of dvrec is comprised of three concurrent processes implemented as separate threads of execution:

- Sampling /dev/dsp and inserting samples into data queue.

- Popping samples from data queue and writing them to disk.

- Monitoring process of other threads and updating database.

dvreport

A report documenting the status of the various software components of DVRAS may be produced using the dvreport software. Should there be sufficient new samples, quick-looks will be generated. dvreport supports the following options:

- max-quicklook:	Limit the number of quick-look pages produced.

- nasty:		Do not reduce process priority.

- debug:		Produce additional diagnostic information.

Under normal circumstances dvreport is run automatically by the crontab, however it may also be invoked directly at any time. Unless –nasty the process reduces its priority to level 19. Combined quick-look files are stored in the /tmp folder, but this folder is intermittently cleared. Thus, the files are moved to /home/dvras/work/combined quicklooks_60days/ and only cleared after 60 days with crontab instructions. The combined quick-look generated file can be seen on Figure 2, when the dvreport script is executed on the crontab.

Figure 2. Combined quick-look report.

dvmail

dvmail oversees the processing of control messages. Before even parsing the messages, they must be authenticated to ensure that they originate from an authorised source. Authentication is by means of a public-key signature. Two emails are sent daily by the DVRAS dvmail script with combined quick-look report attachment.

database

The database tables summarised in Figure 2.3 contain data relating to the samples accumulated by system. These tables are filled automatically by the DVRAS code modules and may be appropriately updated by the user. An additional table, quick-look, which maintains a counter used for automatic numbering of quick-looks, is used internally by the system, and should not require user intervention. The replication of fields between various tables is intentional and facilitates cross-table database queries.

Table 1. Schema of data table in database.

| Field | Description | Unit |
| --- | --- | --- |
| filename | Sample data file name |  |
| time | Sample start time | s |
| station | Station code |  |
| channels | Number of channels |  |
| sfreq | Sampling frequency | Hz |
| neglect | Data validity |  |
| quicklook | Quicklook number |  |
| comment | Textual description of sample |  |

Table 2. Schema of statistics table in database.

| Field | Description | Unit |
| --- | --- | --- |
| time | Sample start time | s |
| station | Station code |  |
| channel | Channel number |  |
| mean | Average digitised sample value |  |
| power | Average power | dB |
| max | Maximum digitised sample value |  |
| min | Minimum digitised sample value |  |
| f1 | Power spectrum first moment (average frequency) | Hz |
| f2 | Power spectrum second moment (average frequency) | Hz |

Table 3. Schema of email table in database.

| Field | Description | Unit |
| --- | --- | --- |
| email | Email address |  |
| admin | Recipient of system messages |  |
| report | Recipient of daily report |  |

Table 4. Schema of schedule table in database.

| Field | Description | Unit |
| --- | --- | --- |
| time | Start time | s |
| duration | Sample length | s |
| sfreq | Sampling frequency | Hz |
| nbyte | Bytes per sample |  |